产品目录

直放

PA10001500_33(2W)

PA11001260_40(10W)

PA12001260_40(10W)

PA12001600_40(10W)

PA12681624_40(10W)

PA14201454_27(0.5W)

PA15001700_50(100W)

PA19301990_45(32W)

PA21102170_34(2W)

PA869960_30(1W)

PA18051990_30(1W)

PA21102170_47(60W)

PA24002480_40(10W)

PA24002500_50(100W)

PA26002700_37(5W)

PA26002700_45(32W)

PA340470_33(2W)

PA450600_33(2W)

PA57005900_42(15W)

PA57005850_50(100W)

TDD 双向放大器

TDD24002500_33(2W)

TDD24002500_27(0.5W)_1T2R

TDD24002500_40(10W)

TDD34003600_37(5W)

TDD750790_43(20W)

TDD890935_37(5W)

LTE18801920_40(10W)

LTE23002400_40(10W)

LTE25802620_40(10W)

移频放大器

TDDSF335345_40(10W)

PA10001500_33(2W)

Operating Frequency：1000MHz～1500MHz

Transmit Gain)：13 ± 1dB

Small Signal Gain Flatness：≤2.0dB

Gain stability：± 1.50dB

Launch Output：	33dBm (2W @COFMD modulation)

Power Consumption：Average current 1.5A @ DC 12V

ON/OFF Control：3.3v High Level

Dimensions：	90 x 48.5 x 14 (mm)

Operating Temp.：	-20℃ ～ +60℃

Weight：	150g

PA11001260_40(10W)

Frequency Range：1100～1260MHz

Launch Output：≥40dBm

Gain：≥40dB

Small Signal Gain Flatness：±1dB

Launch Power Input：Min. 0dBm

Input VSWR：≤2:1

Efficiency：≥20%

Harmonics:≥60dBc

Input/Output impedance：50 Ohm

Supply Voltage：12V-14V

Operating Temp. ：-20℃～ +60℃

Dimensions：120*58*20(mm)

PA12001260_40(10W)

Frequency Range:	1200 ～ 1260 MHz

Launch Output：≥40dBm

Gain：≥40dB

Small Signal Gain Flatness：Peak to Peak  2dB

Launch Power Input：Min. 0dBm

Input VSWR：≤2:1

Efficiency：≥20%

Harmonics:≥60dBc

Input/Output impedance：50 Ohm

Supply Voltage：12V-14V

Operating Temp.：-40℃ ～ +60℃

Dimensions：120*58*20(mm)

PA12001600_40(10W)

Frequency Range:	1200 ～ 1600 MHz

Launch Output：≥40dBm

Gain：≥40dB

Small Signal Gain Flatness：Peak to Peak  2dB

Launch Power Input：Min. 0dBm

Input VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Consumption: 2.5A @ DC 12V

ON/OFF Control：ON/Low Level or GND;OFF/High Level

Operating Temp.：-40℃ ～ +60℃

Dimensions：100*60*20(mm)

PA12681624_40(10W)

Frequency Range:	1268 ～ 1624 MHz

Launch Output：40±2 dBm（Full band）

Gain：40±2.5 dB

Small Signal Gain Flatness：Peak to Peak ≤3dB

Harmonics：9K～1GHz: ≤-36dBm/30KHz

Input VSWR：≤1.80

Supply Voltage：＋27V DC

Power Consumption: ≤2.5 A @27V

Operating Temp.：-10℃ ～ +55℃

Dimensions：178*91*24(mm)

PA14201454_27(0.5W)

Frequency Range:	1420 ～ 1454 MHz

Launch Output：≥27dBm

Gain：49±1dB

Small Signal Gain Flatness：Peak to Peak  1dB

Launch Power Input：-18±2.0dBm

Output Shoulder Ratio：≥-40dBc

Input/Output impedance：50 Ohm

Supply Voltage：＋7.2~18Vdc

Power Consumption: ≤5W (0.35 A @12V)

Operating Temp.：-20℃ ～ +60℃

Dimensions：60*35*20(mm)

Weight：	100g

PA15001700_50(100W)

Frequency Range:	1500 ～ 1700 MHz

Launch Output：≥50dBm

Gain：50 ± 1 dB

Small Signal Gain Flatness：Peak to Peak  2dB

Launch Power Input：Min. 0dBm

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Supply: DC 24V

ON/OFF Control：ON/Low Level or GND;OFF/High Level

Operating Temp.：-25℃ ～ +60℃

Dimensions：1950*150*25(mm)

PA19301990_45(32W)

Operating Frequency：1940MHz～1990MHz

Launch Output：32W（CW）

Transmit Gain：40± 1 dB

Small Signal Gain Flatness：Peak to Peak  2dB  Over Freq Band

Gain Stability：± 1.50dB Over Temp.

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Consumption：: 3A @28V

RF Connectors）： SMA Female

Operating Temp.：-20℃ ～ +60℃

I/O Map：D-Sub 9 Pin

Dimensions：100*100*30(mm) excluding connectors

PA21102170_34(2W)

Frequency Range:	2110 ～ 2170 MHz

Launch Output：≥34dBm

Small Signal Gain：20dB Minimum - 23dB Maximum

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Supply: Minimum 12.0V, Maximum 16.5V

DC Current：1.8A Maximum

RF Connectors： SMA Female

Operating Temp.：-20℃ ～ +60℃

Dimensions（excluding connectors）：58*47*15(mm)

PA869960_30(1W)

Frequency Range:	869 ～ 960 MHz

Launch Output： 30dBm

Small Signal Gain：15±1dB

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Supply: Minimum 12.0V, Maximum 16.5V

DC Current：400mA Maximum

RF Connectors： SMA Female

Operating Temp.：-20℃ ～ +60℃

Dimensions（excluding connectors）：58*47*15(mm)

PA18051990_30(1W)

Frequency Range:	1805 ～ 1990 MHz

Launch Output： 30dBm

Small Signal Gain：14±1dB

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Supply: Minimum 12.0V, Maximum 16.5V

DC Current：400mA Maximum

RF Connectors： SMA Female

Operating Temp.：-20℃ ～ +60℃

Dimensions（excluding connectors）：58*47*15(mm)

PA21102170_47(60W)

Operating Frequency：2110MHz～2170MHz

Launch Output：60W（CW）

Transmit Gain：45± 1 dB

Small Signal Gain Flatness：Peak to Peak  2dB  Over Freq Band

Gain Stability：± 1.50dB Over Temp.

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Consumption：: 6.5A @28V

RF Connectors）： SMA Female

Operating Temp.：-20℃ ～ +60℃

I/O Map：D-Sub 9 Pin

Dimensions：100*120*30(mm) excluding connectors

PA24002480_40(10W)

Frequency Range:	2400 ～ 2480 MHz

Launch Output：≥40dBm

Gain：40 ± 1 dB

Small Signal Gain Flatness：Peak to Peak  2dB

Launch Power Input：Min. 0dBm

Gain Stability)：± 1.50dB

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Consumption: 2.5A @ DC 12V

Indicating lamp : Red

Operating Temp.：-40℃ ～ +60℃

Dimensions：110*120*25 (mm)

PA24002500_50(100W)

Frequency Range:	2400 ～ 2500 MHz

Launch Output：≥50dBm

Gain：≥50dB

Small Signal Gain Flatness：± 1.5 dB

Launch Power Input：Min. 0dBm

Input VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Consumption: 12A @ DC 28V

Efficiency: ≥25%

RF Connnector：N Female (50Ω)

Operating Temp.：-20℃ ～ +60℃

Temp. Protection：Switch Off @≥62℃

ON/OFF Control：Default ON, Switch Off @ Low Level

Dimensions：236*156*35 (mm)

PA26002700_37(5W)

Operating Frequency：2600MHz～2700MHz

Launch Output：37 dBm（CW）

Transmit Gain：35± 1 dB

Small Signal Gain Flatness：Peak to Peak  2dB  Over Freq Band

Gain Stability：± 2.0dB Over Temp.

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Consumption：: 0.7A @28V

RF Connectors）： SMA Female

Operating Temp.：-20℃ ～ +60℃

I/O Map：D-Sub 9 Pin

Dimensions：100*100*30(mm) excluding connectors

PA26002700_45(32W)

Operating Frequency：2600MHz～2700MHz

Launch Output：> 45 dBm（CW）

Transmit Gain：40± 1 dB

Small Signal Gain Flatness：Peak to Peak  2dB  Over Freq Band

Gain Stability：± 1.5 dB Over Temp.

Input/Output VSWR：≤1.5:1

Input/Output impedance：50 Ohm

Power Consumption：: 4.4A @28V

RF Connectors）： SMA Female

Operating Temp.：-20℃ ～ +60℃

I/O Map：D-Sub 9 Pin

Dimensions：100*100*30(mm) excluding connectors

PA340470_33(2W)

Frequency Range:	340 ～ 470 MHz

Launch Output：≥33dBm(2W @COFDM Signal)

Gain：≥13dB

Small Signal Gain Flatness：≤2.0dB

Input/Output impedance：50 Ohm

Power Consumption: 1.5A @ DC 12V

RF Connnector：SMA Female (50Ω)

Operating Temp.：-20℃ ～ +70℃

ON/OFF Control：Switch ON @ High Level 3.3v

Dimensions：90*48.8*12.5 (mm)

Net Weight: 150g

PA450600_33(2W)

Frequency Range:	450 ～ 600 MHz

Launch Output：≥33dBm(2W @COFDM Signal)

Gain：≥13dB

Small Signal Gain Flatness：≤2.0dB

Input/Output impedance：50 Ohm

Power Consumption: 1.5A @ DC 12V

RF Connnector：SMA Female (50Ω)

Operating Temp.：-20℃ ～ +70℃

ON/OFF Control：Switch ON @ High Level 3.3v

Dimensions：90*48.8*12.5 (mm)

Net Weight: 150g

PA57005900_42(15W)

Frequency Range:	5700 ～ 5900 MHz

Launch Output：≥42dBm(15W)

Gain：≥44dB

Small Signal Gain Flatness：Peak to Peak 2dB

Launch Power Input: -2 dBm

In/Out VSWR：≤1.5:1

Power Consumption: 2A @ DC 24V /42 dBm

RF Connnector：SMA Female (50Ω)

Operating Temp.：-10℃ ～ +40℃

Dimensions：185*45*20 (mm)

PA57005850_50(100W)

Frequency Range:	5700 ～ 5850 MHz

Launch Output：≥50 dBm(100W)

Gain：50±1 dB

Small Signal Gain Flatness：± 1.5 dB

Launch Power Input: 0 dBm

Power Consumption: ≤13A @ DC 28V

RF In Connnector：SMA Female (50Ω)

RF Out Connnector：N Female (50Ω)

Operating Temp.：-20℃ ～ +50℃

Dimensions：220*120*25 (mm)

TDD24002500_33(2W)

Frequency Range: 2.4～2.5GHz

Working Pattern: TDD

Launch Output:	33dBm (2w)

Launch Power Input: Min: 5 dBm～Max: 17 dBm

Switch On Power Input (Customized):	Max: -5 dBm

Transmit Gain:	19 dB typical

Receive Gain: 16 dB typical

Small Signal Gain Flatness: ±1dB

Noise Figure: ＜3.0dB

Switch time delay: ＜1us

AGC Control	:Enable

RF Connectors: SMA-K-(50Ω)

Operating Temp.: -25℃ ～ +55℃

Indicating lamp: Green / Red

Power Consumption: 1A @ DC 12V

Dimensions:	92*82*24(mm)

Weight: 400g

TDD24002500_27(0.5W)_1T2R

Frequency Range: 2.4～2.5GHz

Working Pattern: TDD

Launch Output:	27dBm (0.5w @ 2 way Total Output Power)

Launch Power Input: Min: 5 dBm～Max: 16 dBm

Transmit Gain:	15 dB typical

Receive Gain: 14 dB typical

Small Signal Gain Flatness: ±1dB

Noise Figure: ＜6.5dB

Switch time delay: ＜1us

RF Connectors: SMA-K-(50Ω)

Operating Temp.:	-25℃ ～ +55℃

Indicating lamp:	Green / Red

Power Consumption: 0.5A @ DC 12V

Dimensions:	90*60*20(mm)

Weight(g):	300g

TDD24002500_40(10W)

Frequency Range: 2.4～2.5GHz

Working Pattern: TDD

Launch Output:	40dBm (10w)

Launch Power Input:	Min: 5 dBm～Max: 20 dBm

Transmit Gain:	22 dB typical

Receive Gain: 18 dB typical

Small Signal Gain Flatness: ±1dB

Noise Figure: ＜3.0dB

Switch time delay: ＜1us

AGC Control	: Enable

RF Connectors: SMA-K-(50Ω)

Operating Temp.: -25℃ ～ +55℃

Indicating lamp: Green / Red

Power Consumption: 4A @ DC 28V

Dimensions:	150*170*24 (mm)

Weight(g): 900g

TDD34003600_37(5W)

工作频段(Frequency Range):	3.4～3.6GHz

工作模式(Working Pattern):	TDD

发射功率(Launch Output):Max 37dBm (5w)

输入功率(Launch Power Input):	Min:5 dBm～Max:25 dBm

发射增益(Transmit Gain): 12 dB typical

接收增益(Receive Gain): 14 dB typical

驻波 VSWR：≤1.8

噪声系数(Noise Figure):	＜3.5dB

开关延时(Switch time delay): ＜1us

射频接头(RF Connectors): N-K-(50Ω)

工作温度(Operating Temp. ): -25℃ ～ +55℃

指示灯(Indicating lamp): Green / Red

功耗(Power Consumption): 2A @ DC 28V

尺寸(Dimensions):	128*156*27(mm)

净重(Weight):	1000g

TDD750790_43(20W)

工作频段(Frequency Range):	750～790 MHz

工作模式(Working Pattern):	TDD

发射功率(Launch Output):Max 43dBm (20w)

输入功率(Launch Power Input):	Min:10 dBm～Max:28 dBm

发射增益(Transmit Gain): >12 dB

接收增益(Receive Gain): 15 dB typical

噪声系数(Noise Figure):	＜3.0 dB

开关延时(Switch time delay): ＜1us

射频接头(RF Connectors): SMA-K-(50Ω)

工作温度(Operating Temp. ): -25℃ ～ +55℃

指示灯(Indicating lamp): Green / Red

功耗(Power Consumption): 1.3A @ DC 28V

尺寸(Dimensions):	140*65*20(mm)

TDD890935_37(5W)

工作频段(Frequency Range): 890-935MHz

工作模式(Working Pattern): TDD

发射功率(Launch Output): >37dBm

输入功率(Launch Power Input):	Min:10 dBm～Max:27 dBm

发射增益(Transmit Gain): >13dB

接收增益(Receive Gain): >10dB

噪声系数(Noise Figure):	＜3.0 dB

开关延时(Switch time delay): ＜1us

射频接头(RF Connectors): SMA-K-(50Ω)

工作温度(Operating Temp. ): -25℃ ～ +55℃

功耗(Power Consumption): 5A @ DC 12V

尺寸(Dimensions):	86*66*20(mm)

LTE23002400_40(10W)

工作频段(Frequency Range):	2.3～2.4GHz

工作模式(Working Pattern):	TDD

发射功率(Launch Output):Max 41dBm (10w)

输入功率(Launch Power Input):	Min:5 dBm～Max:25 dBm

发射增益(Transmit Gain): 50 dB typical

接收增益(Receive Gain): 12 dB typical

驻波 VSWR：≤1.5

开关延时(Switch time delay): ＜1us

射频接头(RF Connectors): SMA-K-(50Ω)

工作温度(Operating Temp. ): -25℃ ～ +55℃

功耗(Power Consumption): 8A @ DC 28V

尺寸(Dimensions):	286*187*21(mm)

净重(Weight):	2000g

此规格书参数同样适合用于LTE1880～1920MHz & LTE2580～2620MHz

TDDSF335345_40(10W)

工作频段(Frequency Range):	335～345 MHz/ 2462MHz

工作模式(Working Pattern):	TDD

发射功率(Launch Output): 40dBm (10w) @340MHz

输入功率(Launch Power Input):	Min:7 dBm～Max:17 dBm (@2462MHz)

发射增益(Transmit Gain): >30 dB

接收增益(Receive Gain): 10 dB typical

噪声系数(Noise Figure):	＜3.0 dB

开关延时(Switch time delay): ＜1us

射频接头(RF Connectors): SMA-K-(50Ω)

工作温度(Operating Temp. ): -25℃ ～ +55℃

指示灯(Indicating lamp): Green / Red

功耗(Power Consumption):3A @ DC 12V

尺寸(Dimensions):	87*66*22(mm)

特点：将2.4G wifi信号变频至340MHz 低频，利用低频的绕射能力，来增强数据在复杂环境下的传输（如楼梯上下层之间、地下车库、煤矿、巷道等）。